This component uses the mcpwm capture unit of esp32. It creates PWM capture object. Its purpose is to create a capture object that scans for different PWM width signals (registered at the timme of creation). When ever it finds a pulse, the callback registered with the driver calculates the pulse width and pushes to a queue. The associated tasks then checks against the set widths. If there is a match it informs the upper level component about the detection by calling the callback. The task is separate for each object so that there is never a fail to xQueuesendfromisr since it has no blocking time. The assocaiated tasks have wait time option which is set  to portmax delay

The object has both class data and instance data. The class data is shared by all the instances. The class data includes the pulse width array that is the standard for comparison. Similarly there is a capture timer object array, that is shared by multiple objects


How to use

In order to use it the Scan manager object is created